Conjugate Heat Transfer in Rayleigh-Benard Convection in  a Square Enclosure with Sinusoidal Bottom Wall

Project report submitted to

Visvesvaraya National Institute of Technology, Nagpur

In partial fulfillment of the requirements for the award of

the degree

Bachelor of Technology

In

Mechanical Engineering

by

Vivek Vijay Potdar (BT15MEC052)

under the guidance of

Dr. T. B. Gohil

Department of Mechanical Engineering

Visvesvaraya National Institute of Technology

Nagpur 440 010 (India)

2015-19

©Visvesvaraya National Institute of Technology (VNIT) 2019

### Department of Mechanical Engineering

### Visvesvaraya National Institute of Technology, Nagpur

# Declaration

I, Vivek Vijay Potdar , hereby declare that this project work titled “ Conjugate Heat Transfer in Rayleigh-Benard Convection in a Square Enclosure with Sinusoidal Bottom Wall ” is carried out by us in the Department of Mechanical Engineering of Visvesvaraya National of Technology, Nagpur. The work is original and has not been submitted earlier whole or in part for the award of any degree/diploma at this or any other Institution/University.

Date:

Vivek Vijay Potdar

(BT15MEC052)

# Certificate

This is to certify that the project titled “ Conjugate Heat Transfer in Rayleigh-Benard Convection in a Square Enclosure with Sinusoidal Bottom Wall ”, submitted by, Vivek Vijay Potdar in the partial fulfillment of the requirements for the award of the degree of Bachelor of Technology in Mechanical Engineering, VNIT Nagpur. The work is comprehensive, complete and fit for the final evaluation.

Dr.T. B. Gohil

Assistant  Professor

Dept. of Mechanical Engineering

VNIT, Nagpur

Head, Department of Mechanical Engineering

VNIT, Nagpur

Date:

# ACKNOWLEDGEMENT

I have a great pleasure to express my deep sense of gratitude towards my respectful advisor  Dr. T. B. Gohil , Assistant Professor, Department of Mechanical Engineering, Visvesvaraya National of Technology, for his patience, motivation,  immense knowledge and continuous support throughout my project work. These words are formal thanks, which cannot express the real depth of feeling and affection that I have towards him. Beside my advisor , I would like to thank the rest of my project panel professors for their insightful comments and encouragement.

My sincere thanks also goes to the M.Tech and Ph.D students for guiding me and sharing  knowledge throughout my project.

Vivek Vijay Potdar

( BT15MEC052 )

Abstract

In the present work, Conjugate natural convection-conduction heat transfer in a square cavity with a finite wall thickness is studied numerically. Three walls of the cavity are taken to be of zero thickness, while the bottom wall of the cavity is thick, with a finite thermal conductivity. In conjugate heat transfer, Heat conduction in a solid is implicitly coupled with heat convection in viscous fluid flow. The main focus of study is effect of conduction in a hot thick wavy wall on a steady, laminar natural convection in square cavity. The effect of different governing parameters like Rayleigh number, thermal conductivity ratio, wall thickness ratio, Amplitude, frequency of undulation on Nusselt number is studied, while keeping the Prandtl number constant. The range within the governing parameters considered are the Rayleigh number (5 × 103 ≤ Ra ≤ 106), the wall-to-fluid thermal conductivity ratio (0.5 ≤ Kr ≤ 10), the ratio of wall thickness to its height (0.02 ≤ D ≤ 0.4), amplitude (0.06≤ A ≤0.08), No. of undulation (0≤ n ≤2).The study is completed , analyzed and compared between two different geometries of thick bottom wall namely plane wall and wavy wall. It is observed that the fluid flow, temperature distribution pattern and heat transfer rate in the cavity can be controlled by above governing parameters.

List of Figures

Contents

Figure 3.1. Case I - Square cavity with plane wall……………………………………………….10

Figure 3.2. Case II - Square cavity with Sinusoidal wall………………………………………11

Figure  3.3.  Square Enclosure with Plane Bottom Wall (D=0.2)………………………….18

Figure  3.4. Square Enclosure with Sinusodal Bottom Wall (D=0.4, A=0.08, n=1….19

Figure  3.5.  Square Enclosure with Sinusodal Bottom Wall (D=0.1, A=0.06, n=2)..19

Figure  3.6. Grid Independency Study………………………………………………………………..20

Figure  3.7. (a) Isotherm plot from research paper of Habibis saleh and Isak Hashim     (b) Isotherm Results obtained in present study…………………………………………………..22

Figure  4.1. Isotherms (left) and Streamlines(right) at Ra =105 and Kr =1…………...25

Figure  4.2. Isotherms (left) and Streamlines(right) at Ra =105 and D =0.4………….27

Figure  4.3. Variations of Nu with Ra for different D at Kr =1……………………………...28

Figure  4.4. Variations of Nu with Ra for different Kr at D = 0.4…………………………..29

Figure  4.5. Isotherms (left) and Streamlines(right) at Ra =105 , Kr =1, A=0.06, n=2...

…………………………………………………………………………………………………………………………31

Figure 4.6. Isotherms (left) and Streamlines(right) at Ra=106,D=0.1, A=0.06,n=2…....

…………………………………………………………………………………………………………………………33

Figure  4.7. Isotherms (left) and Streamlines(right) at Ra =105 , D = 0.4,  Kr =5,  n=2

…………………………………………………………………………………………………………………………35

Figure 4.8. Isotherms(left) and Streamlines(right) at Ra=106,D=0.2,Kr=0.5, A =0.08.

…………………………………………………………………………………………………………………………36

Figure  4.9. Variations of Nu with Ra for different D at Kr =1, A =0.06, n=2…………37

Figure  4.10. Variations of Nu with Ra for different Kr at D =0.1, A =0.06, n=2…….38

Figure  4.11. Variations of Nu with Ra for different A at D=0.4 , Kr =5, n=2………… 39

Figure  4.12. Variations of Nu with Ra for different n at D=0.2 , Kr =0.5, A = 0.08…40

List of Tables

Contents

Table 1. The values of L for different Amplitudes	12

Table 2.  Boundary Conditions	15

Table 3. Comparison of Nusselt Number between present study and other works	22

Table 4. Maximum Value of Streamfunction ψmax for different wall thickness parameter D  for plane wall	24

Table 5. Maximum Value of Streamfunction ψmax for different thermal conductivity ratio Kr  for plane wall 	26

Table 6. Maximum Value of Streamfunction ψmax for different wall thickness parameter D for Sinusoidal wall	30

Table 7. Maximum Value of Streamfunction ψmax for different thermal conductivity ratio Kr for Sinusoidal wall	32

Table 8. Maximum Value of Streamfunction ψmax for different Amplitude A for Sinusoidal wall	34

Table 9. Maximum Value of Streamfunction ψmax for different Number of Undulations n  for Sinusoidal wall	34

NOMENCLATURE

𝑑,𝐷                                  Wall thickness and dimensionless wall thickness parameter

𝑔                                     Gravitational acceleration

Kr                                    Thermal conductivity ratio

𝑘                                      Thermal conductivity

ℓ                                      Width and height of enclosure

Average Nusselt number

Ra                                   Rayleigh number

𝑇                                      Temperature

𝑢, v                                  Velocity components in the 𝑥 and  𝑦-directions

𝑥, 𝑦                                  Space coordinates

h                                      Convective heat transfer coefficient

A                                     Convective  heat transfer Area

Q                                     Heat transfer rate

n                                      Direction vector

n                                      Number of undulation

A                                      Amplitude of wavy wall

Gr                                   Grashof Number

Pr                                    Prandtl Number

Cp                                    Specific Heat Capacity at Constant Pressure

L                                      Characteristic Length

Greek  Symbols

α                                      Effective thermal diffusivity

β                                      Coefficient of thermal expansion of fluid

µ                                      Viscosity of the fluid

ν                                      Kinematic viscosity of the fluid

ρ                                      Density of the fluid

ψ                                     Value of the streamfunction in the the  enclosure

Subscripts

w , f                                 Wall and Fluid

h , c                                 Hot and Cold

ref                                   Reference value

diff                                  Difference

p                                      Constant Pressure

∞                                    Infinity

max                                Maximum Value

TABLE OF CONTENTS

ABSTRACT……………………………………………………………………………………………………….i

LIST OF FIGURES…………………………………………………………………………………………….ii

LIST OF TABLES……………………………………………………………………………………………..iii

NOMENCLATURE……………………………………………………………………………………………iv

CHAPTER 1.  INTRODUCTION………………………………………………………………………….2

  - Background……………………………………………………………………………………………..2

  - Rayleigh-benard Convection…………………………………………………………………….2

  - Conjugate Heat Transfer…………………………………………………………………………..3

  - Bossinesq Approximation………………………………………………………………………...4

  - Applications…………………………………………………………………………………………….5

CHAPTER 2.  LITERATURE SURVEY…………………………………………………………………7

2.2  Outcomes of Literature Survey…………………………………………………………………8

CHAPTER 3.  NUMERICAL METHODOLOGY……………………………………………………10

  - Problem Description………………………………………………………………………………10

  - Ansys Fluent………………………………………………………………………………………….13

  - Solution Method…………………………………………………………………………………….13

  - General Structure of  Ansys Fluent………………………………………………………….14

  - Boundary Conditions……………………………………………………………………………..15

  - Governing Equations……………………………………………………………………………...16

  - Mesh Generation……………………………………………………………………………………18

  - Grid Independency Study……………………………………………………………………….20

  - Validation Study…………………………………………………………………………………….22

CHAPTER 4.  RESULTS  &  DISCUSSION………………………………………………………….24

  - Case I :- Plane Bottom Wall……………………………………………………………………..24

    - Effect of  Governing Parameters  on Thermal and Flow Field…………24

      - Effect of  Wall Thickness Parameter  D………………………………..24

      - Effect of  Thermal Conductivity Ratio Kr………………………….....26

    - Effect of  Governing Parameters  on Nusselt Number……………………..28

      - Effect of  Wall Thickness Parameter  D………………………………..28

      - Effect of  Thermal Conductivity Ratio Kr……………………………..29

  - Case II :- Plane Bottom Wall……………………………………………………………………30

    - Effect of  Governing Parameters  on Thermal and Flow Field………….30

      - Effect of  Wall Thickness Parameter  D………………………………..30

      - Effect of  Thermal Conductivity Ratio Kr………………………….....32

      - Effect of Amplitude of Sinusoidal Wall A…………………………….34

      - Effect of Number of  Undulation n………………………………………34

    - Effect of  Governing Parameters  on Nusselt Number……………………..37

      - Effect of  Wall Thickness Parameter  D………………………………..37

      - Effect of  Thermal Conductivity Ratio Kr……………………………..38

      - Effect of Amplitude A  and Number of  Undulation n……………39

CHAPTER 5. CONCLUSION……………………………………………………………………………..42

REFERANCES…………………………………………………………………………………………………43

1. INTRODUCTION

# INTRODUCTION

## 1.1  Background

The convection and conduction are two main phenomenon generally observed in many the heat transfer problems. In many fields of science, including Engineering , Architecture, Medical science and also in human body, the diverse applications of interaction of these two phenomenon is commonly observed. These are main mechanisms of heat transfer within fluid and solid . Many scientists mathematically studied these two phenomenons and tried to explain them in mathematical formula. Before computers came into existence , Newton proposed less accurate method and  gave formula to calculate the heat  flux from a isothermal wall in convective heat transfer.

Qw = h A (Tw -T∞)

As the value of heat transfer coefficient  h mainly depends on wall temperature distribution, there is significant amount of difference is observed between theoretical  and experimental results. So to do precise  formulation of convective heat transfer problems which involve solid-fluid interaction, the proper coupling of the fluid and solid temperature field should be done at their interface.

Similarly for conduction, Fourier proposed theory of conduction in which he shows that amount of heat flux in a direction is directly proportional to the temperature gradient in respective direction.

Qw    =

where constant  of proportionality is given as thermal conductivity. so these two formulae  form  the basic of heat  transfer problems. In conjugate heat transfer problem, these two phenomenon coupled with each other at interface of two regions.

1.2  Rayleigh-benard convection

“ Reyleigh-benard convection” is one of the types of natural convection. In 1900,the French physicist ,Henry benard conducted a simple experiment regarding natural convection in  fluid heated from bottom surface and observed that  a beautiful hexagonal regular pattern of convection cells is developed. These cells are called as “Benard cells” .

Later lord Rayleigh  successfully  analyzed convection. Rayleigh considered boundary conditions in which vertical velocity component and temperature distribution are vanishes at top and bottom boundaries. This convection is solely due to the temperature gradient which leads to formation of density gradient in the fluid from top to bottom. This creates the buoyancy force which tends to move hot fluid in upward direction.  While Gravity is also trying to pull cold fluid in downward direction. But gravitational force is opposed by the viscous damping force .so the balance between these two forces is studied using non-dimensional parameter “Rayleigh Number”. After critical value of Rayleigh number, gravitational force dominates, thus instability sets and convection cells are formed. Thus buoyancy and gravity are responsible to creation of convection cells.

These cells are formed in alternate way in rotation sense from clockwise to anticlockwise. Cells are metastable in nature that a small fluctuation in governing conditions does not affect them.

1.3  Conjugate Heat Transfer

Conjugate heat transfer refers to the method of computing thermal distribution for heat conduction in solids, along with simultaneous convective heat transfer in associated fluid domain.  Conjugate heat transfer mainly deals with the heat transmission between a solid and a fluid body flowing inside or over it as a result of the interaction of two objects. To do the conjugation, coupled boundary conditions  are available for both the fluid and solid cell zones, where either of the zones, may contain heat sources. In conjugate heat transfer problems generally containing two or more subdomains with different phenomena describing by different differential equations. After solving the differential equation problem in each subdomain , these solution are conjugated. This coupling gave the heat flux and temperature distribution across the interface. The final outcomes of such a problems will gave the heat flux and temperature and velocity distribution in the domains.

For example, the heat transfer between a solid plate and a fluid flowing over it, is a conjugate heat transfer problem, because the heat transfer inside the solid is governed by the elliptical Laplace equation or by the parabolic differential equation, while the elliptical Navier-Stokes equation or by the parabolic boundary layer equation governs the heat transfer inside the fluid. Both domains are initialized with their corresponding properties and separate boundary conditions are provided for both the domains except at the interface. The governing partial differential equations of each subdomain is solved explicitly. A common thermal boundary condition is imposed at interface which coupled solutions of fluid and solid domains. This coupling transfer the thermal properties and characteristics across domains , which eliminates the need of explicitly providing heat transfer coefficient across the fluid-solid interface.

1.4  Bossinesq Approximation

In convective heat transfer, the fluid properties are strongly influenced by variations in temperature. In such problems , all  the fluid properties are calculated using updated temperature value at current iterations . In case of natural convection, density is varies with temperature which causes buoyancy driven flow  within  fluid. The momentum equation and Energy equation of fluid flow is coupled and solved by considering  suitable relationship of  variation of density with  temperature.  General expression for this relationship is  given by

β =

The  bossinesq Approximation is used to define the relationship between density and temperature. It states  , for small temperature difference ,there is linear relationship between density and temperature.

β =

The approximation further explains that the variation in inertia of fluid particles is insignificant but gravity is sufficiently strong enough to create appreciably difference in specific weight . Hence, when density variation is small and within limits, density can be treated as constant in unsteady and convection terms, but as a variable in  body force  or gravitational term in momentum equation.

Body force term = (ρ-ρref) ∙ g = - ρ β g (T-Tref)

There is a constraint on the implementation of bossinesq approximation as it is valid only for those cases where value of  ρeff  ≪ 1. The additional term ρeff is   known as  “Effective density”. If value of  ρeff  ≫ 0.4 , then  flow is considered as compressible flow.

△ ρdiff  =  ρ - △  ρ  = [ρ – ρ β (T –Tref)]

ρeff  =  = 1 - β (T –Tref)

1.5  Applications

There are several engineering applications of the conjugate heat transfer are present in many areas of the Science. Starting from simple examples in the 1960s, the conjugate heat transfer methods have become a more powerful tool for modeling and investigating nature phenomena and engineering systems in various areas ranging from  thermal goods treatment and food processing to aerospace and nuclear reactors to, from complex procedures in medicine to atmosphere/ocean thermal interaction in meteorology, and from relatively simple units to multistage, nonlinear processes.

This convection problem  in the present study has attracted a great deal of attention from researchers because of its presence in both nature and industrial applications. In nature, the convection cells formed from air rising above sunlight-warmed land or water are a major feature of all weather systems. Convection is also seen in the rising plume of hot air from fire, oceanic currents, and sea-wind formation. A very common industrial application of natural convection is free air cooling without the aid of fans: this can happen on small scales (computer chips) to large scale process equipment.

2. LITERATURE  SURVEY

# LITERATURE SURVEY

1.         Habibis saleh and Isak hashim studied the conjugate heat transfer in rayleigh-benard convection in a square cavity with a finite wall thickness. In study, the horizontal bottom wall of cavity having finite thickness and thermal conductivity, while other walls having zero thickness. In problem, finite and uniform temperature difference is applied with hot bottom wall and cold top wall keeping side wall insulated. They studied the effect governing parameters like wall thickness ratio(D), Rayleigh Number(Ra), wall to fluid thermal conductivity ratio(Kr) on temperature and velocity distribution. They also studied the effect above parameters on solid-fluid interface Nusselt number. Staggered grid with Marker And Cell (MAC) method were used to solve the Problem. Grid independency test was performed on the basis of Nusselt number. The isotherms and streamlines are plotted and studied varying the one parameter while keeping other constant. Variations of average fluid Nusselt number is plotted with Rayleigh number for different wall thickness ratio and thermal conductivity ratio . Variations of Nusselt number with different wall thickness ratio at different thermal conductivity ratio is also plotted. It is observed that  fluid flow development, circulation strength of flow, heat transfer through interface can be controlled by varying  governing parameters.

The strength of flow circulation is much higher for thin walls. Number of contra rotative cells  can be controlled with parameters.

2.       D. A. KAMINSKI  and  C. Prakash  studied steady, laminar, natural convection  flow with conduction in one of vertical wall of square cavity. One of vertical wall is heated uniformly  while other wall is cold and remaining two horizontal  walls are insulated.  They studied the effect of vertical wall conduction on temperature variation at  fluid –solid interface, fluid flow development , variations of local   heat  flux  at  interface. The SIMPLER algorithm were used during computation of problem. They also plotted isotherms and streamtraces  for one and two dimensional wall conduction model. The study is done on Effect of wall thickness on flow field with help of isotherm and streamlines plot. The Governing parameters are used in mixed non-dimensional form such as  the conduction ratio(KwL/Kt) , non-dimensional form of streamfunction (ѱmaxL2/ν) and Grashof  number (Gr). The variation of  local fluid-solid interface temperature  and local heat  flux at interface is plotted at  different Grashof  number  with variation of conduction ratio keeping Prandtl number constant. The overall Nusselt  number is calculated  and tabulated for different  Grashof number in range of 103  to 107  with varying conduction ratio in range of  5 to ∞ for one and two dimensional wall  conduction and compared  the results with lumped parameter approach and error is calculated.  The correlation was obtained between Grashof  and Nusselt  number. It is  observed that overall Nusselt number is increases with conduction ratio and Grashof number. The temperature variations across interface increase for particular conduction ratio as Grashof number increases. In prediction of overall Nusselt number lumped parameter model  gives slight under- prediction results as compared  to one and two dimensional heat conduction model.

3.                M. Hribersek  and  G. kuhn  solved conjugate problem same as solved by Kaminski and prakash  but with boundary- domain integral method.  He solved Navier-stokes equation with vorticity- velocity formulation and calculate overall Nusselt number. Isotherms and streamlines are plotted with different Grashof number (Gr) and thermal conductivity ratio (k). The interface temperature variation  and interface heat  flux  variations are plotted. He  compared  overall Nusselt  number with FVM method for different  Gr and k values.

4.               L. Adjlout , O. Imine,  A. Azzi, M. Belkadi   studied  the effect of a hot wavy wall of a laminar natural convection in an inclined square cavity with differential heating. All wall are of zero thickness. Temperature difference applied to a pair of opposite wall keeping other pair insulated. The effect of different governing parameters like Rayleigh number (Ra), Amplitude(A),Number of undulation(n) on temperature distribution, overall Nusselt number is studied  The isotherms and streamlines are plotted with variation of rotation angle of cavity (θ) for one and three undulations or crests.  It is  observed Temperature pattern and fluid flow  can be controlled by rotation angle (θ) and No. of undulations (n).

2.2  Outcomes of Literature Survey

  - A considerable amount of attention has been given to investigate the effect of  Rayleigh number, Thermal conductivity Ratio, Wall Thickness Ratio on interface average Nusselt number, fluid flow development, temperature distribution across the cavity  by adopting different  computational and numerical procedure  and compared with experimental results.

  - Many authors changed the position of conduction wall  and studied the effect of it on various outcomes with varying the governing parameters. The change in geometry of conduction wall is studied with considering zero thickness. Various relationships were proposed between average Nusselt number and different parameters.

3. NUMERICAL  METHODOLOGY

NUMERICAL  METHODOLOGY

3.1  Problem Description

In the present problem, we considered two geometries for study of conjugate heat transfer in square cavity. In study,  the problem is considered as two dimensional heat conduction- convection problem.

Case I :-

The schematic diagram of case 1 is shown in Figure 3.1. In case 1, we consider plane , solid, impermeable bottom wall having finite thickness d and thermal conductivity kw  while other wall having zero thickness. The bottom surface of  the solid wall is heated at constant temperature Th while top surface of cavity is cooled at constant temperature Tc keeping two vertical surface adiabatic. The cavity between top surface and solid wall is filled with air. Dimensions  of square cavity is taken as  l ×l =1 ×1 m.

Y

Tc

Fluid

X

l

l

(0,0)

Th

d

g

Wall

# Figure 3.1. Case I - Square cavity with plane wall

Case II :-

Figure 3.2 shows the schematic diagram of case 2. Here we considered  bottom wall as wavy wall  having  a sinusoidal nature. The sinusoidal wall is  chosen  as hot wall with different amplitude and two undulations. The wavy wall is solid ,impermeable in nature having thickness d and thermal conductivity kw. Other  surfaces are having  same thermal parameters as in case 1.

The equation of sinusoidal wave of wall is given  as

y =  f (x) =  A ( 1 – cos (2nπx) )

where,    0 ≤ x ≤ 1

Y

Tc

l

Adiabatic  Left  Wall

g

Adiabatic  Right  Wall

Fluid

d

Wall

X

l

Th

# Figure 3.2. Case II - Square cavity with Sinusoidal wall

- The  Governing  Parameters:-

- Rayleigh Number (Ra):-

5 × 103  ≤ Ra ≤ 106

Ra = Gr. Pr = (g .β. (Th – Tc).L3/ ν2) . Pr

2.    Thermal Conductivity Ratio (Kr) :-

0.5 ≤ Kr ≤ 10

Kr =

3.      Wall Thickness Ratio (D):-

0.02 ≤ D ≤ 0.4

D =

4.       Amplitude (A) : -

0.06 ≤ A ≤ 0.08

5.       Number of  Undulation (n) :-

n = 1 and 2

Thermo-physical  Properties of  Air :-

At  300 K,

- Density = ρ = 1.157 Kg/m3

- Dynamic Viscosity = µ = 1.87 × 10-5  N-s/m2

- Kinematic Viscosity = ν =   =1.6162 × 10-5  m2/s

- Thermal Conductivity  = Kf  = 0.02676 W/m-K

- Specific Heat at constant pressure = Cp = 1011.73 J/Kg-k

- Prandtl Number  = =  0.707

For Case 1,

l = L = 1 m.

For Case  2,

l ≠ L

# Table 1. The values of L for different Amplitudes

|  | A = 0.06 | A=0.08 |
| --- | --- | --- |
| n=1 | 1.0316 | 1.0604 |
| n=2 | 1.1273 | 1.2170 |

To  simplify the analysis , following assumptions were made for computing

the  numerical  solution while keeping the flow physics undisturbed.

- The all physical properties  of  fluid is  independent  with  respective to temperature  except  the density is varies  with temperature which supported by bossinesq approximation.

- Similarly, all thermo-physical properties of  solid  and fluid is constant  everywhere in their respective domains.

- Prandtl number of air is assumed as constant.

3.2  Ansys Fluent

There are several commercial as well as open source soft wares are available for CFD analysis. In the present study , the Ansys Fluent is used to simulate the flow  in the system and quantify  the Engineering parameters.

ANSYS FLUENT software contains the wide varieties physical modeling capabilities needed to model flow, heat transfer, turbulence and reactions for industrial applications ranging from air flow over an aircraft wing to combustion in a furnace, from bubble columns to oil platforms, from blood flow to semiconductor manufacturing, and from clean room design to wastewater treatment plants. Special models that give the software the ability to model in-cylinder combustion, turbo machinery, aero acoustics and multiphase systems have served to broaden its reach.

3.3  Solution Method

Solver used :-

Pressure based Type,

Absolute velocity Formulation,

Steady state  ,2-Dimensional solver

With laminar viscous model.

Method Used for Pressure- velocity Coupling :-  SIMPLEC

Spatial Discretization Method :-

The process of descretization involved transformation of         continuous functions, variables into approximate  discrete parts and interpolate variable values. This methods convert Differential equations into Difference equations. The methods employed for these  approximation  can affect both the accuracy and stability of the numerical schemes. Hence special care  must also be taken to ensure that the discretization  handles discontinuous solution gracefully ,following are the discretization schemes are being used for different terms.

Gradient Term – Least Square Cell Based

Pressure Term  - Second Order

Momentum Term – Second Order Upwind

Energy Term – Second Order Upwind

3.4  General Structure of Ansys Fluent :-

Setup

General

Models

Materials

Cell Zone Conditions

Boundary Conditions

Mesh Interface

Solution

Solution Methods

Solution Controls

Monitors

Solution Initialization

Calculation Activities

Results

Graphics

Animations

Plots

Reports

3.5  Boundary  Conditions

In present study, upper surface is cold while the bottom wall is uniformly heated keeping side walls insulated. In both cases Momentum and Thermal Boundary Conditions remains same.

# Table 2.  Boundary Conditions

| Sr.No. | Part name | Part Type | Momentum Boundary Conditions | Thermal Boundary Conditions |
| --- | --- | --- | --- | --- |
| 1. | Top_Wall | wall | Stationary wall & No Slip shear Condition | Constant Temperature= Tc = 300 K |
| 2. | Bottom_Wall | wall | - | Constant Temperature= Th = 301 K |
| 3. | LHS_Fluid | wall | Stationary wall & No Slip shear Condition | Constant Heat Flux = 0 w/m2 |
| 4. | LHS_Solid | wall | - | Constant Heat Flux = 0 w/m2 |
| 5. | RHS_Fluid | wall | Stationary wall & No Slip shear Condition | Constant Heat Flux = 0 w/m2 |
| 6. | RHS_Solid | wall | - | Constant Heat Flux = 0 w/m2 |
| 7. | Fluid_Solid_ Interface | Interface | - | - |
| 8. | Solid_Fluid_ Interface | Interface | - | - |

A Mesh Interface is  created  by connecting Fluid_Solid_ Interface with Solid_Fluid_ Interface with coupled wall condition.

The values of the velocity are zero in the wall region and on the solid-fluid interfaces. The boundary conditions for the temperatures are the following :-

Tw (X,0) = Tc ;                Tf (X,1) = Th ;

;   ;   ;

;	.

3.6    Governing  Equations

In present study, The thermo-physical properties of fluid is assumed to be constant  except the density is varies with temperature .To couple temperature field with flow field , Bossinesq Approximation is invoked  for fluid particles which relates the density with temperature linearly. With bossinesq approximation, density is assumed to constant in unsteady and convective  term but variation of density is considered in body force term.  Under above assumptions , the governing equations of mass ,momentum, and energy for the fluid can be  written as ,

The Mass- Continuity Equation:-

(1)

The Navier- stokes/ Momentum Equation:-

,

(2)

The Energy Equation:-

(3)

And the Energy Equation/ Laplace Equation for  the impermeable wall is given as

(4)

No- slip condition is assumed at solid-fluid interface and all surfaces. The following Non-dimensional parameters used for calculation of properties of medium :

,

.

Calculation of Nusselt Number :-

by Fourier’s law of Heat Conduction,

Heat Flux at Interface =                                                         (A)

by Newton’s law for convection ,

Heat Flux at Interface =  h ∆T                                                               (B)

From A and B,

For ∆T =1,

h = Heat Flux at Interface =

Average Nusselt Number  =  Nu ==

3.7  Grid Generation

Grid generation or meshing is one of the crucial step in  any simulation problem. In Grid generation , we spilt  a large domain into small sub-domains and create mesh. In any numerical simulation, the discretization of governing equations is done in each small sub-domains.

In present problem of conjugate heat transfer , Two main domains as Solid and Fluid are considered. These two domains are coupled by  a common interface named as Solid_Fluid_interface through which  solution of one domain is  coupled to solution of another domain. Two different meshes were created  for each domain  and a single mesh is generated by merging these two meshes. A mesh 100 × 100  is used for fluid domain while mesh of  20 × 100 for solid domain. Total Number of nodes is 12,000 in each mesh. The meshing of both domains are  Structured  and Non-uniform in nature. Mesh is very fine near the walls while course in middle of each domain . Meshing is done with Bi-Geometric mesh Law , having  initial node spacing of 0.001 m with incremetation   of 12% in spacing in both sides. All mesh generation is completed through ICEM-CFD . Figures shows some of the meshes used for simulation.

# Figure  3.3.  Square Enclosure with Plane Bottom Wall (D=0.4)

Figure  3.4. Square Enclosure with Sinusoidal Bottom Wall(D=0.2,A=0.08,n=1)

# Figure  3.5.  Square Enclosure with Sinusodal Bottom Wall (D=0.1,A=0.06,n=2)

Mesh Sensitivity Analysis :-

3.8  Grid Independency Study

In Numerical analysis, the solution and its convergence is largely dependent on fineness of cell and its position in the mesh. The size of each cell in the Mesh affects the outcomes of problem.  So, It is necessary to check whether the solution is sensitive to change in configuration of mesh. Grid Independency Test tells us grid size suitable for numerical analysis. In Grid Independency Test , Deviation of a parameter of solution  is checked by varying the size of Grid.

In present study, Average Nusselt Number is calculated by variation of grid  Size . In test we take grid size of 30, 60, 100, 140 for Fluid domain.

The Test is done for Governing parameters D = 0.1 , Kr = 0.5 , Ra = 105 with  plane bottom wall ,keeping all other thermal-physical boundary conditions and properties of fluid-solid constant. All the governing Equations were remain as it throughout the mesh sensitivity analysis.

The Graph is plotted between Average Nusselt Number and Grid size. Figure shows the Graph     vs. Grid size in X and Y Direction.

# Figure  3.6. Grid Independency Study

It is observed that  for mesh size 100 and above , Average Nusselt number does not show any significant variations. So ,Mesh size 100 is suitable for  numerical computational procedure.

3.9  Validation Study

In order to know the accuracy and limitations of the result obtained by the our method ,we compared the obtained results with Research study done by Habibis Saleh and Isak Hashim. Comparison of average Nusselt number between the present study and other works is done for D=0.4 .

# Table 3. Comparison of Nusselt Number between present study and other works

| Ra → | 105 |  |  | 106 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Kr → | 1 | 5 | 10 | 1 | 5 | 10 |
| Present Study | 1.516 | 3.302 | 3.900 | 1.798 | 4.598 | 5.781 |
| Habibis Saleh and Isak Hashim. | 1.50 | 3.35 | 3.85 | 1.85 | 4.50 | 5.70 |
| % Error | 1.067 | 1.433 | 1.30 | 2.811 | 2.178 | 1.421 |

Comparison of Isotherms for D =0.02  at Ra =105 and Kr =1 :-

- (b)

# Figure  3.7. (a) Isotherm plot from research paper of Habibis saleh and Isak Hashim (b) Isotherm Results obtained in present study.

4. RESULT & DISCUSSION

RESULT & DISCUSSION

The Problem of Conjugate heat transfer in a square Enclosure consists  conduction through solid bottom wall parallel with convection  in fluid in the cavity. In current problem, Top surface is cooled at constant temperature  while bottom wall is heated at constant temperature keeping the other two side walls insulated. The conduction  of solid wall affect the flow circulation  and temperature distribution in the cavity. So, Heat conduction in a solid is implicitly coupled with heat convection in viscous fluid flow. The nature of flow, heat transfer rate, temperature distribution  can be controlled by various governing parameters like Rayleigh number, thermal conductivity ratio, wall thickness ratio, Amplitude, frequency of undulation. The range within the governing parameters considered are the Rayleigh number (5 × 103 ≤ Ra ≤ 106), the wall-to-fluid thermal conductivity ratio (0.5 ≤ Kr ≤ 10), the ratio of wall thickness to its height (0.02 ≤ D ≤ 0.4), amplitude (0.06≤ A ≤0.08), No. of undulation (0≤ n ≤2). Effects of above parameters on Nusselt number is calculated. Effects of two geometries of bottom wall i.e. Plane wall and Sinusoidal Wavy wall are studied , analyzed and compared the obtained results.

4.1 Case I :- Plane Bottom Wall

4.1.1  Effect of  Governing Parameters  on Thermal and Flow Field

4.1.1.1 Effect of Wall Thickness Parameter D

Figure 4.1 illustrates the effects of wall thickness parameter D on the thermal field  and flow fields for Ra =105 and Kr = 1. Figures shows that ,the wall thickness parameter D affect the temperature distribution in solid , fluid as well as flow characteristics. From table we can see that circulation strength of fluid flow is higher for thin solid bottom wall and it goes on decreasing as D increases. This happened due to decrease in the temperature  difference across the cavity.  The fluid near to the hot wall has a lower density than the fluid at top cold wall. The hot fluid experience the buoyancy force so it tends to move upwards and cold fluid tries to go down . This creates the convection cell Known as main cell.

# Table 4. Maximum Value of Streamfunction ψmax for different wall thickness parameter D  for plane wall

|  | Maximum Value of Streamfunction ψmax (×10-4) |
| --- | --- |
| D =0.02 | 6.575806 |
| D =0.1 | 5.430327 |
| D =0.4 | 3.329330 |

D=0.02

D=0.1

D=0.4

Figure  4.1. Isotherms (left) and Streamlines(right) at Ra =105 and Kr =1

From figure we can be seen that, For Small thickness of bottom wall, a main circular cell is formed. As we increases thickness parameter D, the circular main cell becomes elliptical in nature and finally breaks into two   contra –rotative cells. This two cells called as Benard cells.  When fluid goes up , it gets cooled  and flows in right and left planes of the enclosure. This create two successive  benard cells. The cell in clockwise direction ( -ve sign)  formed due to natural circulation , called “main circulation cell” and the second corresponding  counterclockwise (+ve sign) cell is called as “Secondary circulation cell”.

4.1.1.2 Effect of Thermal Conductivity Ratio Kr

Figure 4.2 illustrates the effects of wall Thermal Conductivity Ratio Kr on the thermal field  and flow fields for Ra =105 and D=0.4 . To show the effect we chose three different values for Kr i.e. Kr =0.5, Kr =1 ,Kr =10. From Figures we can see that two Stable contra-rotative cells i.e. clockwise main circulation cell and anticlockwise secondary cells are formed due to considerable high value of wall thickness parameter D.  As we increases  value of Kr , Circulation pattern  is remain unaffected ,but strength of circulation  goes on increases. This phenomenon occurs  due to increase  in wall conduction which results into increase in temperature  gradient  near  the wall ,further leads to increase in convective heat flux.  So it is observed that on increase of thermal conductivity ratio Kr, the convective heat transfer within the cavity becomes  stronger.

# Table 5. Maximum Value of Streamfunction ψmax for different thermal

# conductivity Kr

|  | Maximum Value of Streamfunction ψmax (×10-4) |
| --- | --- |
| Kr =0.5 | 2.499482 |
| Kr =1 | 3.329330 |
| Kr =10 | 5.380897 |

Kr=0.5

Kr=1

Kr=10

Figure  4.2. Isotherms (left) and Streamlines(right) at Ra =105 and D =0.4.

4.1.2  Effect of  Governing Parameters  on Nusselt Number

4.1.2.1 Effect of Wall Thickness Parameter D on Nusselt Number

Variations of average Nusselt number with the Rayleigh  number for different values of wall thickness parameter are shown in Figure 4.3. It is observed from the graph for any value of wall thickness parameter , average Nusselt number increases  as  Rayleigh number increases. This is due to the increase in the Rayleigh number ,increase the convective mode of heat transfer inside the fluid. From the graph,  we can conclude that as wall thickness parameter  D  goes  on increases , the average Nusselt Number goes on decreases.

.

# Figure  4.3. Variations of Nu with Ra for different D at Kr =1.

4.1.2.2  Effect of Thermal Conductivity Ratio Kr on Nusselt Number

Figure 4.4 show variation of the the average Nusselt number  with the Rayleigh number for different value of thermal conductivity ratio Kr . The Nusselt number is increases as thermal conductivity ratio increases. This is due to the  increase in the wall thermal conductivity , heat transfer rate through wall increases. This  increases temperature difference across the cavity ,which motivates the convective heat  transfer through the cavity .

# Figure  4.4. Variations of Nu with Ra for different Kr at D = 0.4.

4.2 Case II :- Sinusoidal Bottom Wall

4.2.1  Effect of  Governing Parameters  on Thermal and Flow Field

4.2.1.1  Effect of Wall Thickness Parameter D

Figure 4.5 shows the effect of the wall thickness parameter D on thermal field and flow field for a sinusoidal bottom wall with two undulations, amplitude  A = 0.06  for different Wall thickness parameter  D at Ra = 105 , Kr =1 .  As we can see that, the parameter D affect the fluid  and  solid temperature and as well as the flow characteristics . From the plots we can observed  that ,due to wavy nature of wall the two contra-rotative cells  are formed at thin wavy bottom wall instead of one main circulation cell as in case of plane bottom wall. As D increases , these cell becomes elliptical and main circulation cell starts  to dominate.  The Circulation strength of  wall of  D = 0.02 is much higher than the wall of D = 0.4. As  D increases , the circulation strength of main cell  decreases.  Due to increase in D, the temperature drop across the solid increases , leads to decrease in interface wall  flux. So heat transfer though the convection mode decrease. This results in decrease in circulation strength.

# Table 6. Maximum Value of Streamfunction ψmax for different wall thickness

# parameter D for Sinusoidal wall

|  | Maximum Value of Streamfunction ψmax (×10-4) |
| --- | --- |
| D=0.02 | 5.128163 |
| D=0.1 | 4.465025 |
| D=0.4 | 1.788775 |

D=0.02

D=0.1

D=0.4

Figure  4.5. Isotherms (left) and Streamlines(right) at Ra =105 , Kr  =1, A=0.06, n=2.

4.2.1.2  Effect of Thermal Conductivity Ratio Kr

The effect of the Thermal Conductivity Ratio Kr on thermal field and flow field for a sinusoidal bottom wall  is shown by figure 4.6  for Ra = 106, D = 0.1,         A = 0.06  and n = 2.  Four different thermal conductivity ratio were selected  : Kr =0.5 , Kr =1, Kr =5, Kr =10 . A small circulation cell is formed in the cavity , because of high Rayleigh number (Ra =106) which increases  convective  heat transfer  through fluid and remains  stable as increase in Kr . Due to increase in Kr  result into increase in heat transfer  through wall . so , heat flux rate across the interface goes on increases  ,which  make convective heat transfer stronger.  The table shows that , circulation  strength of the main cell goes on increases as Kr increases.

# Table 7. Maximum Value of Streamfunction ψmax for different thermal conductivity ratio Kr for Sinusoidal wall

|  | Maximum Value of Streamfunction ψmax (×10-3) |
| --- | --- |
| Kr =0.5 | 1.118388 |
| Kr =1 | 1.275891 |
| Kr =5 | 1.503197 |
| Kr =10 | 1.548791 |

Kr=0.5

Kr=1

Kr=5

Kr=10

Figure 4.6. Isotherms(left) and Streamlines(right) at Ra=106,D=0.1,A=0.06.

4.2.1.3  Effect of Amplitude of Sinusoidal Wall  A

Figure 4.7 shows the effect of  change in Amplitude A  of sinusoidal wall on the thermal field and flow field  for parameter Ra =105 , D = 0.4 , Kr = 5 and n = 2. Two values of A is taken A=0.06, A=0.08 and results compared with Plane wall        (A =0).  Two circulation cells  were created at plane wall ,as we increases the amplitude of wall ,the circulation cells becomes elliptical but there isn’t any drastic changes in the circulation pattern of flow is observed. The increases in the amplitude of Sinusoidal wall , decreases the strength of the circulation of flow.

# Table 8. Maximum Value of Streamfunction ψmax for different Amplitude A for Sinusoidal wall

|  | Maximum Value of Streamfunction ψmax (×10-4) |
| --- | --- |
| A = 0 | 4.963788 |
| A = 0.06 | 3.050293 |
| A = 0.08 | 2.078449 |

4.2.1.4  Effect of Number of Undulations  n

Figure 4.8 shows the effect of   sinusoidal wall on the thermal field and flow field  for parameter Ra =106 , D = 0.2 , Kr = 5 and n = 2. The a main circulation cell is formed with plane wall . On introduction of waviness in the bottom wall ,circulation cell become elliptical and circulation strength decreases. But as we increases number of frequency of waviness , circulation strength starts on increases.

# Table 9. Maximum Value of Streamfunction ψmax for different Number of Undulations n  for Sinusoidal wall

|  | Maximum Value of Streamfunction ψmax (×10-4) |
| --- | --- |
| n = 0 | 10.05457 |
| n = 1 | 8.8861161 |
| n = 2 | 8.994908 |

A = 0

A = 0.06

A = 0.08

Figure  4.7. Isotherms (left) and Streamlines(right) at Ra =105, D=0.4,Kr =5.

n = 0

n = 1

n = 2

Fig  4.8. Isotherms (L) and Streamlines(R) at  Ra=106, D=0.2, Kr=0.5, A =0.08.

4.2.2  Effect of  Governing Parameters  on Nusselt Number

4.2.2.1  Effect of Wall Thickness Parameter D

Figure 4.9 shows the variations of average Nusselt number with the Rayleigh  number for different values of wall thickness parameter are shown in Figure. The graph shows that increase in Rayleigh number , increase Nusselt number across the interface for any particular value of D. Rayleigh number increases the domination of convective heat transfer i.e. increase in value of h at interface because of increase in the buoyancy forces inside the fluid. As increase in the value of wall thickness parameter D , the value of Nusselt number goes on decreases. The increment in the value of D leads to decrease in the temperature difference across the cavity.  So convective heat transfer decreases i.e. decreases in value of  h.

# Figure  4.9. Variations of Nu with Ra for different D at Kr =1, A =0.06, n=2.

4.2.2.2 Effect of Thermal Conductivity Ratio Kr

Figure 4.10 illustrate the Effect of Thermal Conductivity Ratio Kr on Nusselt Number . The graph shows that ,The increases in the wall thermal conductivity and Rayleigh number, increase the Nusselt number . As we know on increases in Thermal conductivity ratio Kr , more heat transfer through wall takes place. Due to this temperature gradient across wall interface increase , results in increase in convective heat transfer coefficient h at the interface. So overall Nusselt number goes on increases with constant characteristic length L and constant wall thermal conductivity Kw  as in Kr.

Figure  4.10. Variations of Nu with Ra for different Kr at D =0.1, A =0.06, n=2.

4.2.2.3   Effect of Amplitude of Sinusoidal Wall A and Number of Undulations  n

The Effect of change in Amplitude A on Nusselt number with varying  the Rayleigh number  is shown by the Figure 4.11  for parameter  D=0.4 , Kr =5, n=2.  It is observed that the increase in  Amplitude  A  decreases the Nusselt number. This  is because as increase in Amplitude of wavy wall , there is increase in heat transfer area at the interface so heat flux rate at interface goes on decreases. Though there is increases in value of L but dominance of decrease in heat flux rate the value of Nusselt number decreases.

The figure 4.12 shows the effect of Number of undulation n on the Nusselt number at  Ra = 105 and Ra  = 106 . From n = 0 (plane wall case ) to n = 1 , the value of Nusselt  number decreases  due to dominance decreases in the  heat flux rate  over increase in characteristics length L . From n =1 to n = 2 , the value of Nusselt number start increases. This is because of significant increase in the characteristics length L  dominates  over the decreases in heat flux rate at interface.

Figure  4.11. Variations of Nu with Ra for different A at D=0.4 , Kr =5, n=2.

Figure  1.12. Variations of Nu with Ra for different n at D=0.2 , Kr =0.5, A = 0.08.

5. CONCLUSION

CONCLUSION

In the present simulation study , the effect of conduction heat transfer in a Sinusoidal bottom wall on Rayleigh-benard Convection in square cavity is studied.  The effects of  different Governing  parameters  i.e. Rayleigh number (Ra) , Thermal conductivity Ratio (Kr) , Wall thickness ratio (D),  Amplitude (A), Number of undulations (n)  are studied. The Detailed computational Results for flow and temperature field  have been presented and analyzed in the form of streamlines and isotherm plots. The effects of governing parameters on Nusselt number  at interface is also  presented in the graphical forms.  The main Conclusions of the  present  simulation study  are as follow.

- The strength of the flow circulation of the fluid is much higher for the thin bottom wall and for the high thermal conductivity Ratio. But  strength  of circulation goes on decreases as thickness of bottom wall increases or thermal conductivity ratio decreases.

- The number of Contrarotative cells and their strength of circulation  can be controlled by the thermal conductivity ratio, wall thickness ratio, Rayleigh number ,amplitude and number of undulations .

- The strength of flow circulation is decreases  on introducing wavy nature to the plain bottom bottom wall.

- The increase in amplitude of wave decreases the circulation strength of fluid in the cavity. The strength of circulation is increases as number of undulation increases.

- The average Nusselt number increases by increasing either the thermal conductivity ratio , the Rayleigh number or number of undulation.

- The increase in the wall thickness ratio or the amplitude of wave, decreases the average Nusselt number.

REFERENCES

- Habibis Saleh and Ishak Hashim , “Conjugate Heat Transfer in Rayleigh-Bénard  Convection in a Square Enclosure”, Hindawi Publishing Corporation The Scientific World Journal Volume 2014, Article ID 786102

- D. A. Kaminski and C. Prakash, “Conjugate natural convection in a square enclosure: effect of conduction in one of the vertical walls,” International Journal of Heat and Mass Transfer, vol. 29, no. 12, pp. 1979–1988, 1986.

- M. Hribersek and G. Kuhn, “Conjugate heat transfer by boundary-domain integral method,” Engineering Analysis with Boundary Elements, vol. 24, no. 4, pp. 297–305, 2000

- N.Wansophark, A. Malatip, and P. Dechaumphai, “Streamline upwind finite element method for conjugate heat transfer problems,” Acta Mechanica Sinica, vol. 21, no. 5, pp. 436–443, 2005.

- L. Adjlout *, O. Imine, A. Azzi, M. Belkadi ,“Laminar natural convection in an inclined cavity with a wavy wall”  ,ELSEVIER International Journal of Heat and Mass Transfer 45 (2002) 2141–2152

- Abram Dorfman and Zachary Renner ,“Conjugate Problems in Convective Heat Transfer: Review”, Hindawi Publishing Corporation Mathematical Problems in Engineering Volume 2009, Article ID 927350

- https://en.wikipedia.org/wiki/Conjugate_convective_heat_transfer

- https://en.wikipedia.org/wiki/Rayleigh-benard_convection